ITC-RT550

无线通信测试仪

产品手册

Version 2.8

2021.08

版权声明

深圳市鑫信腾科技股份有限公司©2021。版权所有，保留一切权利。

未经深圳市鑫信腾科技股份有限公司书面许可，任何人或组织不得以任何形式修改或摘录本文档的任何部分。

Copyright © 2021 ShenZhen System Technology Company Limited. All Rights Reserved.

No part of this manual may be reproduced in any form, written or otherwise, without the express written permission of ShenZhen System Technology Company Limited.

目录

1. 前言

1.1. 产品简介

1.1.1. BR测试功能

1.1.2. EDR测试功能

1.1.3. LE测试功能

1.1.4. CW发射功能

1.1.5. Signal Generator功能

1.1.6. 技术指标

1.2. 选件列表

- 前言

本手册将以简明的方式介绍ITC-RT550无线通信测试仪（以下简称ITC-RT550）使用方法等相关信息。

1.1产品简介

ITC-RT550是由深圳市鑫信腾科技股份有限公司研发、制造的无线通信测试仪。该仪表主要面向蓝牙终端测试，支持BR/EDR LE射频测试，也具备信号源功能，未来可通过升级支持最新蓝牙标准。ITC-RT550具备多个RF端口，支持远程控制，单表测试功能丰富，也可与其他仪表集成，组成大型无线通信测试系统。ITC-RT550可广泛应用于蓝牙终端的研发、生产、认证等领域。

ITC-RT550具备如下应用优势：

- 内部集成射频开关，直接支持1拖2测试

- 1U体积，易于工厂架线或工厂部署

- 提供一键式自动化测试工具，支持GPIB over LAN远程控制

- 支持蓝牙5.1标准，可软件升级最新蓝牙标准

    - BR测试功能

ITC-RT550提供BR射频测试功能，支持如下测量项。

BR发射机测试

| 协议章节 | 测量项 |
| --- | --- |
| RF/TRM/CA/BV-01-C | Output Power |
| RF/TRM/CA/BV-03-C | Power Control |
| RF/TRM/CA/BV-07-C | Modulation Characteristics |
| RF/TRM/CA/BV-08-C | Initial Carrier Frequency Tolerance |
| RF/TRM/CA/BV-09-C | Carrier Frequency Drift |
| RF/TRM/CA/BV-14-C | Enhanced Power Control |

BR接收机测试

| 协议章节 | 测量项 |
| --- | --- |
| RF/RCV/CA/BV-01-C | Sensitivity - single slot packets |
| RF/RCV/CA/BV-02-C | Sensitivity - multi-slot packets |
| RF/RCV/CA/BV-06-C | Maximum Input Level |

    - EDR测试功能

ITC-RT550提供EDR射频测试功能，支持如下测量项。

EDR发射机测试

| 协议章节 | 测量项 |
| --- | --- |
| RF/TRM/CA/BV-10-C | EDR Relative Transmit power |
| RF/TRM/CA/BV-11-C | EDR Carrier Frequency Stability and Modulation Accuracy |
| RF/TRM/CA/BV-12-C | EDR Differential Phase Encoding |
| RF/TRM/CA/BV-15-C | EDR Guard Time |
| RF/TRM/CA/BV-16-C | EDR Synchronization Sequence and Trailer |

EDR接收机测试

| 协议章节 | 测量项 |
| --- | --- |
| RF/RCV/CA/BV-07-C | EDR Sensitivity |
| RF/RCV/CA/BV-08-C | EDR BER Floor Performance |
| RF/RCV/CA/BV-10-C | EDR Maximum Input Level |

    - LE测试功能

ITC-RT550提供LE射频测试功能，支持如下测量项。

LE发射机测试

| 协议章节 | 测量项 |
| --- | --- |
| RF-PHY/TRM/BV-01-C | Output Power |
| RF-PHY/TRM/BV-05-C | Modulation Characteristics, uncoded data at 1 Ms/s |
| RF-PHY/TRM/BV-06-C | Carrier frequency offset and drift, uncoded data at 1 Ms/s |
| RF-PHY/TRM/BV-09-C | Stable Modulation Characteristics, uncoded data at 1 Ms/s |
| RF-PHY/TRM/BV-10-C | Modulation Characteristics, uncoded data at 2 Ms/s |
| RF-PHY/TRM/BV-11-C | Stable Modulation Characteristics, uncoded data at 2 Ms/s |
| RF-PHY/TRM/BV-12-C | Carrier frequency offset and drift, uncoded data at 2 Ms/s |
| RF-PHY/TRM/BV-13-C | Modulation Characteristics, LE Coded (S=8) |
| RF-PHY/TRM/BV-14-C | Carrier frequency offset and drift, LE Coded (S=8) |
| RF-PHY/TRM/BV-15-C | Output power, With Constant Tone Extension |
| RF-PHY/TRM/BV-16-C | Carrier frequency offset and drift, uncoded data at 1 Ms/s, CTE |
| RF-PHY/TRM/BV-17-C | Carrier frequency offset and drift, uncoded data at 2 Ms/s, CTE |

LE接收机测试

| 协议章节 | 测量项 |
| --- | --- |
| RF-PHY/RCV/BV-01-C | Receiver sensitivity, uncoded data at 1 Ms/s |
| RF-PHY/RCV/BV-06-C | Maximum input signal level, uncoded data at 1 Ms/s |
| RF-PHY/RCV/BV-07-C | PER Report Integrity, uncoded data at 1 Ms/s |
| RF-PHY/RCV/BV-08-C | Receiver sensitivity at 2 Ms/s |
| RF-PHY/RCV/BV-12-C | Maximum input signal level at 2 Ms/s |
| RF-PHY/RCV/BV-13-C | PER Report Integrity at 2 Ms/s |
| RF-PHY/RCV/BV-14-C | Receiver Sensitivity, uncoded data at 1 Ms/s, Stable Modulation Index |
| RF-PHY/RCV/BV-18-C | Maximum input signal level, uncoded data at 1 Ms/s, Stable Modulation Index |
| RF-PHY/RCV/BV-19-C | PER Report Integrity, uncoded data at 1 Ms/s, Stable Modulation Index |
| RF-PHY/RCV/BV-20-C | Maximum input signal level at 2 Ms/s, Stable Modulation Index |
| RF-PHY/RCV/BV-21-C | PER Report Integrity at 2 Ms/s, Stable Modulation Index |
| RF-PHY/RCV/BV-25-C | Receiver sensitivity, LE Coded (S=2) |
| RF-PHY/RCV/BV-26-C | Receiver sensitivity, LE Coded (S=8) |
| RF-PHY/RCV/BV-27-C | PER Report Integrity, LE Coded (S=2) |
| RF-PHY/RCV/BV-30-C | PER Report Integrity, LE Coded (S=8) |
| RF-PHY/RCV/BV-31-C | Receiver sensitivity, LE Coded (S=2), Stable Modulation Index |
| RF-PHY/RCV/BV-32-C | Receiver sensitivity, LE Coded (S=8), Stable Modulation Index |
| RF-PHY/RCV/BV-33-C | PER Report Integrity, LE Coded (S=2), Stable Modulation Index |
| RF-PHY/RCV/BV-36-C | Receiver sensitivity at 2 Ms/s, Stable Modulation Index |
| RF-PHY/RCV/BV-37-C | PER Report Integrity, LE Coded (S=8), Stable Modulation Index |

    - CW发射功能

ITC-RT550支持CW测试功能。

CW测试

| 参数 | 描述 |
| --- | --- |
| 频点 | 2402~2480MHz |
| 采样时间 | 0.1~3ms |
| 调制类型 | GFSK、DQPSK、8DPSK |
| GFSK下测试指标 | 频偏（均值）、功率、最大频偏、最小频偏 |
| DQPSK及8DPSK下测试指标 | Peak功率、RMS功率、Peak DEVM、RMS DEVM |

    - Signal Generator功能

ITC-RT550支持信号生成功能。

Signal Generator功能

| 参数 | 描述 |
| --- | --- |
| 信源类型 | CW、10101010、11110000、PRBS9、PRBS15 |
| 频点 | 2402~2480MHz |
| 调制类型 | GFSK、DQPSK、8DPSK |
| 调制指数 | 0.28~0.35 |
| 发射功率 | -10~-120dBm |

*ITC-RT550已支持LE信号生成功能，详情请与技术支持联系。

    - 技术指标

      - 系统指标

系统指标

| 条目 | 描述 |
| --- | --- |
| 蓝牙标准 | 5.1 |
| 蓝牙制式 | BR，EDR，LE（1M uncoded，2M uncoded，LR，可选支持CTE） |
| 射频接口 | 4 (2 in/out, 1 in, 1 out) |
| EUT接口 | USB，2 wire |
| USB | 2 |
| LAN | 1 |
| HDMI | 1 |
| Audio | 1 in/out |
| Ref | 1 (10 MHz) |
| 尺寸（长*宽*高）mm | 370*220*44（不含可拆卸脚垫） |

      - 发射机指标

发射机指标

| 条目 | 描述 |
| --- | --- |
| 频率范围 | 2.4 GHz ~ 2.5 GHz |
| 频率分辨率 | 1kHz |
| 频率精度 | 0.2ppm |
| 功率范围 | RF Out：-120dBm ~ -10dBm RF COM：-120dBm ~ -15dBm |
| 功率精度 | ±0.6dB |
| 功率分辨率 | 0.1dB |
| VSWR | <1.2 |
| GFSK 调制指数 | 0.25 ~ 0.5 |
| PSK 调制精度 | <2% RMS DEVM |

      - 接收机指标

接收机指标

| 条目 | 描述 |
| --- | --- |
| 频率范围 | 2.4GHz ~ 6GHz |
| 频率分辨率 | 1kHz |
| 频率精度 | 0.2ppm |
| 功率范围 | -60dBm ~ 30dBm |
| 功率精度 | ±0.6dB |
| 功率分辨率 | 0.1dB |
| VSWR | <1.2 |
| GFSK 频偏测量范围 | 500kHz |
| PSK 接收EVM | <2% RMS DEVM |

  - 选件列表

选件列表如下：

选件列表

| 条目 | 描述 |
| --- | --- |
| 硬件 |  |
| ITC-RT550-001 | 必选：ITC-RT550硬件平台 |
| ITC-RT550-002 | 必选：ITC-RT550射频模块 |
| 软件 |  |
| ITC-RT550-BR01 | 可选：支持BR信令及测试 |
| ITC-RT550-EDR01 | 可选：支持EDR信令及测试 |
| ITC-RT550-BLE01 | 可选：支持BLE 1M PHY测试 |
| ITC-RT550-BLE02 | 可选：支持BLE 2M PHY测试 |
| ITC-RT550-BLE03 | 可选：支持BLE LR测试 |
| ITC-RT550-BLE04 | 可选：支持CTE |

*更加详细的选件搭配方案请与销售代表联系。

探索6C质造科技